ENGINEERING RELEASE NOTES

S3 NT 4.00 Driver For Trio3D/2X Rev. B or C (V. 3.30.10)

## Recipient

S3 Internal

## Purpose

This release addresses a requirement made by Microsoft WHQL.  It changes an NT registry setting so that, in those cases where a Trio3D2X board is equipped with 8MB but only 4MB can be used for frame buffer (modes, DDRAW, etc.), the Display Properties utility will still display the full 8MB in the “Display Type” Adapter Information.   This, plus the update of the version information, is the only change between driver 3.30.09 and 3.30.10.

## Products Supported

Trio3D/2X

## Language Support

English only.

## Software Requirements

This release requires Microsoft NT4.00 service pack 3, at a minimum.

## Additions and Changes

- This version supersedes version 3.30.09.  Version information updated to 3.30.10.

## Resolutions Supported

- Resolution, color depth, and refresh rate support is dependent on the BIOS used with the driver.  The driver will specifically remove support for some modes according to the following pruning rules for Trio3D/2x:

- No 32bpp modes are offered (24bpp modes are available)

- Resolution 320x240 is only available at 8bpp

- Resolution 1280x1024x24bpp: if refresh rate is 75Hz or 85Hz, and memory width is 32-bit (not 64), the mode is pruned (blocked).

- Resolution 1600x1200x15bpp: if memory width is 32-bit, the mode is pruned.

- In general, if the UseDDCInfo NT registry key is set to 1 (TRUE), the driver will use any EDID information returned by an attached DDC CRT monitor, and will prune modes that are not supported by the monitor.

## Applications/Utility Components

- There are no applications/utility components included with this release.

©Copyright 1999 S3 Incorporated. All rights reserved. If you have received this document from S3 Incorporated in electronic form, you are permitted to make the following copies for business use related to products of S3 Incorporated: one copy onto your computer for the purpose of on-line viewing, and one printed copy. With respect to all documents, whether received in hard copy or electronic form, other use, copying or storage, in whole or in part, by any means electronic, mechanical, photocopying or otherwise, is not permitted without the prior written consent of S3 Incorporated, P.O. Box 58058., Santa Clara CA 95052-8058. S3, S3 ON BOARD, S3d (design and word), Trio and ViRGE are registered trademarks of S3 Incorporated. The S3 Corporate Logo, Sight. Sound. Speed., Trio64V2, Savage, Savage3D, Savage4, DuoView, and Streams Processor are trademarks of S3 Incorporated. Other trademarks referenced in this document are owned by their respective companies. The material in this document is for information only and is subject to change without notice. S3 Incorporated reserves the right to make changes in the product design without reservation and without notice to its users.

## Installation Instructions

- Use the standard method for installing NT 4.0 drivers: install from Settings page of ControlPanel/DisplayProperties. The installation disk for this release should include the following files:

- VERSION.TXT

- README.TXT

- S3T3D2X.INF

- S3MINI.SYS

- S3T3D2X.DLL

## Resolved Issues

- No Clarify report

- Description:  Microsoft WHQL (driver certification) has required that we report the full adapter memory size in the DisplayProperties page, even though we may in some cases be limited to using only half that amount for modes, DirectDraw, etc.

- Resolution:  The S3inc/HardwareInformation registry key will be assigned the full adapter memory size.  Driver operations and memory reported to DirectDraw, etc. remain limited to the usable memory.  The specific configurations being addressed by this fix are Trio3D2X 8MB boards where only 4MB is available for normal non-texture display memory.

## Known Limitations

There is no ICD support in this release, and ICD support has been discontinued for Trio3D/2x.

SQA Engineering Release Comments: Status as of Date: 03/25/99

- This version of software product has been in test for less than 48 hours and HAS passed a limited suite of acceptance testing for the following controllers: Trio3D/2X (362 Rev. B or C).

- Regression of all open issues HAS been completed.

Issues Fix Verified:

CRXXXXX 	Microsoft WHQL (driver certification) has required that we report the full adapter memory size in the DisplayProperties page, even though we may in some cases be limited to using only half that amount for modes, DirectDraw, etc.

Disparities with Developer's Release Note:

- From the "Resolutions Supported" section, item #4 should be "Resolution 1600x1200x16bpp" not 15bpp. The attached Release Note has been edited to reflect the changes above.