80C167 Cross-assembler. Version 1.11.

User's guide.

Copyright (c) 1996-1997 by Cascode Ltd, S-Peterburg

------------------------------------------------------------------------------

Table  of  Contents

------------------------------------------------------------------------------

1.  Introduction

2.  System requirements

3.  Description

4.  Invocation

5.  Input specification

6.  Assembler control

7.  Operands

8.  Assembler directives

9.  SAB 80C167 Instruction set

10.  Error messages

------------------------------------------------------------------------------

1. Introduction

------------------------------------------------------------------------------

Assembler SAB80C167 is used to obtain executable files for  the

SAB80C167 SIEMENS  processor. The  source file  for Assembler  is

that in ASCII format, analogous to the SIEMENS Assembler.  Source

text may  be divided  into several  separate files  which will be

compiled together.   On the  output will  be hexadecimal  file in

the Intel standard ( Intel Hex Format ) ready to the execution.

------------------------------------------------------------------------------

2. System requirements

------------------------------------------------------------------------------

Assembler  is  presented  by  file  ASM167.EXE.  For the normal

operation of Assembler an  IBM PC compatible computer  is needed.

And a free space on a disk 3 times greater than the total  volume

of source text.

------------------------------------------------------------------------------

3. Description

------------------------------------------------------------------------------

Assembler is a double-pass  translator with the possibility  of

compilation of a  several separate files  together to obtain  one

output file ready to  execute in the microprocessor.  Output file

is created in a standard  hexadecimal format thus it's size  will

be  not  greater  than  64  K.  So  there is no possibility of an

intersegment transfer  and reference  with labels.  Inside of the

translated file there is a possibility of using INCLUDE operator.

It gives the possibility  of translating several files  together.

Maximum number of simultaneously translation files are 25.

Assembler is processing input files with 2 passes:

1-st pass       reading of source files, sintax check,

creating of a file of errors and table of

labels.

2-nd pass       code generation and creation of a listing

file.

Assembler  receives  an  input  file  in  ASCII format with the

expansion  .ASM,  which  can  be  used  as a default. The maximum

string length is 255 characters.

As for the output files Assembler can produce :

a) Executable file in  hexadecimal format. The default  expansion

is .HEX.  Having several  input files  the output  file is the

only one.

b) Error  file in  ASCII format.  The default  expansion is .ERR.

Error messages  are also  always given  on a  screen. And  the

error file is also the  only one. This file always  created in

current directory.

c) Listing  file is  in ASCII  format. The  default expansion  is

.LST. Listing file is  created always separately for  the each

input file. At the  end of a listing  of a main file  (name of

which is given in a command line) the table of defined  labels

can be given. This file always created in current directory.

------------------------------------------------------------------------------

4. Invocation

------------------------------------------------------------------------------

Starting assembler the command line looks like follows :

asm167 src_file [dst_fife] [-keys]

To get the information about the command line type :

asm167 -?

The only name of the input  file is needed, it may be  only one

name. All the rest input files are included with INCLUDE operator.

After the name of a main  file the name of the executable  output

file may  stay. Then  the keys  are going  for the  adjusting the

modes of translator. All these keys are actual only for the  file

in a command line.  For the included files  the modes are set  in

each concrete file. The mode that is set in the main src_file has

the priority to that in a command line.

Description of a command line.

a) src_file  Name of the input file. Default expansion is .ASM.

b) dst_file  Name of the output file. If no  name of the output

file is specified and there is no key of cancelling

the output file then the output file will be

created with the name of an input file with .HEX

expansion.

c) keys      Keys of command line, which must be preceded by dash

"-". Only file names are premised.No cathalogue names.

The following keys are defined :

-?       Information about the command line

-O<file> Mode of output file generation is on ,

the name of output file is set - <file>.

The default is - O. If no name is speci-

fied then the file with src_file name

and .HEX expansion is created. Only file

names without cathalogue names are possible.

-NO      Switches off the mode of output file

generation.

-P<file> Generation of a listing file is On and

the name of a listing file is set to

<file>. As a default listing is given on

a screen. If no name is specified then

a file with src_file and .LST expansion

is generated. Only file names without

cathalogue names are possible.

-NP      Listing file generation is Off.

(The default value is -NP).

-E<file> Error file generation with the name <file>.

As a default the error messages are outputted

to the screen. If no name is specified then

the file with the name src_file and the .ERR

expansion is created.Only file names without

cathalogue names are possible.

-NE      Error file generation is Off.

(The default value is -NE).

-G       Mode of paGination of a listing file.The

default number of strings per page is 60.

(The default value is -G).

-NG      Listing paGination is Off.

-Lnnn    Sets number of strings per listing page.

nnn is a decimal value in a range 20...255.

If nnn is not specified then it is  taken

as 60.

-Wnnn    Number of characters in a listing string.

nnn is a decimal value in a range 60...255.

If nnn is not specified then it is  taken

as 120.

-S       Adds to the listing file the table of labels.

(The default value is -S).

-NS      Cancels the -S key.

-C       Casesensitive mode, default.

-NC      Cancels the case sensitive mode for the names

of variables and directives.

------------------------------------------------------------------------------

5. Input file specification

------------------------------------------------------------------------------

Input  file  specification  is  traditional.  Each  string  can

contain label,  one assembler  instruction or  directive and have

symbol  ";"  at  the  end,  that  means  the  end  of  a   string

interpretation and the beginning of a comment. The general  input

string format is as follows :

[Label[:]]   [Instruction|Directive]   [;Comment]

where

Label       Identifier, which is string of characters not more than 31

and having the value of a current code pointer.

The following symbols are forbidden :

".", ",", "'", " ", ";" , ":". The label before assembler's

instruction must finished by symbol ":"

Instruction Any possible instruction of 80C167.

Without operands , with one , two or tree operands.

(See Sect.7 and 9.)

Directive   Any possible directives of  data definition or translation

control. (See Sect.6 and 8.)

Empty strings are valid.

An example :

; Translation mode settings

NOSYMBOLS                          ; Comment

NOERRORPRINT                       ; Mode directive

DATE  '10-10-94'                   ; Mode directive

; Variables definition

LAB1      DB   VARBYTE                       ; Directive

; definition

; of variable

INCLUDE  part2.asm

; Programs definition

EVEN

proc1     PROC                               ; Subroutine

EINIT                              ; Without operands

BSET      ABIT                     ; One operand

LAB2:     AND       R0,#0H                   ; Two operands

BFLDL     0FF0CH, #4 , #6          ; Tree operands

RETI

ENDP

END

------------------------------------------------------------------------------

6. Assembler control

------------------------------------------------------------------------------

All the directives  of compilation control  are actual only  to

the current  file. For  the main  file (that  in command  line ),

the directives mentioned in  a file have priority.  The following

are defined :

1. OBJECT [file_name]   |   NOOBJECT

Permits/forbids output file generation. If file_name is

specified then output file will have this name.

Default:  OBJECT src.HEX.

Command line: -O<file>   |   -NO

2. PRINT [file_name]   |   NOPRINT

Permits/forbids listing file generation. If file_name is

specified then listing file will have this name, if not then

the name will be src_file.LST.

Default: -NP and listing is outputted to the screen.

Command line: -P<file>   |   -NP

3. ERRORPRINT [file_name]   |   NOERRORPRINT

Permits/forbids error file generation. If file_name is

specified then error file will have this name, if not then

the name will be src_file.ERR.

Default: -NE and messages are outputted to the screen.

Command line: -E<file>   |   -NE

4. LIST   |   NOLIST

Permits/forbids appending strings in listing file.

Default: LIST

5. SAVE   |   RESTORE

Save/restore the mode of LIST/NOLIST in stack. Maximum depth

of the stack is 8.

6. PAGING   |   NOPAGING

Permits/forbids pagination of listing file.

Default:  PAGING.

Command line: -G  |  -NG

7. PAGELENGTH nnn

Sets the number of strings in page of listing file.

Default or when nnn is not specified is 60.

Available range 20...255.

Command line: -Lnnn.

8. PAGEWIDTH nnn

Sets the number of characters in listing string.

Default or when nnn is not specified is 120.

Available range 60...255.Longer strings will be cut.

Command line: -Wnnn.

9. EJECT (EJ)

Start new page in listing file.

10. TITLE 'ccc'

String ccc opens each page. Length of string ccc must be less

than 60 characters. This string defined second line in each

page.

11. DATE 'DD-MM-YY'

Defines date in a header string    of a listing file.If not

specified then system date is given.

12. SYMBOLS   |   NOSYMBOLS

Permits/forbids reference table in listing file.

Default: SYMBOLS

Command line: -S   |   -NS

13. CASE   |   NOCASE

Permits/forbids case sensitivity for the directives or

variables names.

Default: CASE

Command line: -C   |   -NC

------------------------------------------------------------------------------

7. Operands

------------------------------------------------------------------------------

Detailed definition of addressing  types and operands is  given

in "Siemens SAB80C167 User's Manual".

------------------------------------------------------------------------------

8. Assembler directives

------------------------------------------------------------------------------

Assembler  directives  are  used  for  the  translation process

control  and  do  not  generate  any  code.  Directives  of   the

following types are used :

a) directives of variables and constants definition

( DB, DW, DDW, DBIT, DS, DSW, DSDW, EQU );

b) directives of translation address control

( ORG, EVEN, ALIGN );

c) directives of subroutine definition

( PROC, ENDP, END );

d) directive INCLUDE;

a) directives of variables and constants definition

1. [ Name ]  DB   init, [init]

Initialization 1 or  more bytes in  memory. If "init"  defines

string enclosed in ","   then string is loaded in  memory byte

after byte.

2.  [ Name ]  DW   init, [init]

Initialization of 2 or more bytes in memory. Init may be number,

constant or label.

3.  [ Name ]  DDW  init, [init]

Initialization of 4 or more bytes in memory. Init may be number,

constant or label.

4.  [ Name ]  DS   n

Reservation of n bytes of memory without initialization.

5.  [ Name ]  DSW  n

Reservation of 2*n bytes of memory without initialization.

6.  [ Name ]  DSDW n

Reservation of 4*n bytes of memory without initialization.

7.  [ Name ]  EQU  n

Defines 2-byte constant with name Name.

8.  [ Name ]  DBIT bitaddr

Defines variable containing bit address. As a bitaddr

may be another bit address, SFR.n, GPR.n., n always in

DECIMAL. Also the bitaddr can be in following format:

xx.n, where xx - 8-bit address of byte, n - bit number

in byte.

Directives of address translation control :

1.  ORG  nnnn

Sets the current translation address to nnnn.

Default address is 0000h.

2.  EVEN

Aligns translation address to word boundary.

If OK makes nothing.

3.  ALIGN  align_type

Aligns translation address to the value of align_type.

WORD - word boundary (analogous to EVEN);

DWORD - double word boundary;

PAGE - page boundary;

Directives of subroutine definition :

1.  proc_name   PROC

Defines subroutine with proc_name name. All the defined

subroutines are only NEAR.

2.  ENDP

Finishes definition of the last defined subroutine.

3.  END

End of translation of current file. All the definitions

after this directive are ignored.

INCLUDE directive :

1.  INCLUDE file_name

In  specified  place  file_name  file  is translating. All the

settings  of  a  current  file  are  not  spreader to included

file which must have his own settings.

------------------------------------------------------------------------------

9. SAB 80C167 Instruction set

------------------------------------------------------------------------------

Assembler is interpreting  al the assembler's  mnemonics, defi-

ned for the SAB80C167.  The detailed description of  the instruc-

tions is given in 'SIEMENS SAB80C167 User's Manual'.

------------------------------------------------------------------------------

10.  Error messages

------------------------------------------------------------------------------

In  the  Assembler  a  powerful  mechanism  of error situations

processing is realized, giving about 50 error messages. In   case

of error situation the message is given on the screen and written

in the file of errors. All the mistakes are divided into    fatal

and ordinary. In case of fatal error, for example:   insufficient

memory for assembling, reading-writing file error, the process of

assembling is  terminated. In  case of  an   ordinary error,  for

example: syntax,   redefining label, the  error  message  appears

on the screen and the process of assembling is going on.

Fatal errors:

1. Insufficient memory for assemble.

Accesable memory must be increased probably delete TSR program.

2. Too many files to compile.

Only 25 source files can be compiled together. May be some

files can be merged together.

3. Error reading file.

4. Error writing file.

5. Too many files in cmd.

In a command line only one input and one output file can be

specified.

6. Invalid switch.

Invalid key is specified in a command line. To get help on

a command line syntax type: asm167 -?

7. Too many labels.

Only 500 identifiers can be specified in a source text.

Ordinary Errors

1. Overflow LIST stack.

The maximum stack  depth for the switches LIST/NOLIST

is 8.

2. Underflow LIST stack.

The coupleness of SAVE/RESTORE operands is broken.

3. Not digit symbol in number.

4. Expected symbol `'`.

5. File not found.

The wrong file name is specified in INCLUDE operand.

6. Not defined filename.

No file name in INCLUDE operand.

7. Immediate data must be 00-FFh.

The data must be from this range.

8. Immediate data must be 0000-FFFFh.

The data must be from this range.

9. Illegal operand.

Common mistake with conflicting operands.

10. Too long string.

The string of a text must be not greater than 255 characters.

11. Out of code space.

No possibility to overcome segment of 64 Kb.

12. Superfluous operand.

13. Immediate data must be 0-7FFFh.

The data must be from this range.

14. Immediate data must be 0-3FFFh.

The data must be from this range.

15. Constant name not defined.

No name is specified when defining a constant.

16. Bit address name not defined.

No name is specified when defining a bit address.

17. Illegal ALIGN type.

In  ALIGN instruction set only : WORD, DWORD, PAGE operands

are possible.

18. PROC name not defined.

19. Only one directive or instruction in the string.

See section 5.

20. Redefined label.

Label can be specified only one time.

21. Source operand not defined.

22. TRAP out of range.

TRAP vector may be in 0-127 range.

23. Operand must be register.

24. 16 bit operand expected.

25. 8 bit operand expected.

26. Target operand must be Rwi.

The first operand must be GPR.

27. Target operand not defined.

First operand is not defined.

28. Immediate data must be 0-7.

The data must be from this range.

29. Operand must be [Rw0]-[Rw3].

An operand may be only index register [R0], [R1], [R2], [R3].

30. Operand must be bit form.

An operand must be bit address.

31. Target operand must be register.

First operand must be register.

32. Direct JUMP address expected.

An  address of direct transfer is expected as an operand.

33. Segment must be 0-3.

At intersegment transfer the segment may be 0-3.

34. First operand must be segment.

First operand must be segment 0-3.

35. Offset address out of range.

In transfer instruction, the transfer address must be not far

than 127 bytes from transfer instruction.

36. First operand must be condition.

37. Immediate data must be 0-15.

The data must be from this range.

38. 16 bit first operand expected.

39. 8 bit second operand expected.

40. Operand not defined.

41. Syntax error.

The word can not be recognized as a instruction, directive or

correct label name ( label must be finished by symbol ":").

------------------------- << *** End of file *** >> --------------------------