GRID systems

Recommended system to use: Window8

(http://msdn.microsoft.com/en-us/evalcenter/jj554510.aspx).

reasons:

1) performance of the kernel (optimized, smaller, more efficient kernel)

2) IPMI can still be used, which is a requirement for data center admin

setup IPMI with static IP (xxx.yyy.zzz.aaa), verify connectivity by using http://xxx.yyy.zzz.aaa

user/password = ADMIN / ADMIN

Window7/Windows2008R2

- install Windows 7 with monitor connected to onboard VGA

- network driver from:

- ftp://ftp.supermicro.com/CDR-X9-UP_1.22_for_Intel_X9_UP_platform/Intel/LAN/

- install GRID driver. verify in device manager.

- install VNC enterprise http://www.realvnc.com/download/enterprise/ , set VNC auth password. verify connectivity to the system.

- use onboard jumper to disable VGA (if onboard VGA is left on with Windows 7, it prevents the WDDM GPU drivers from loading)

- reboot

(the problem is that you can still connect to the IPMI for basic administration, there will be no display of the SBIOS, since the onboard VGA is disabled)

Windows 8/Windows Server 2012

- install the OS

- network driver are installed by default

- leave the onboard VGA. Windows 8 has built-in software WDDM drivers for the onboard VGA that don't prevent WDDM GPU drivers from loading.

- so the IPMI remains fully functional to remote into SBIOS

Finish install:

remote into windows with VNC, force use resolution control panel

- for each GPU

    - select, force attach to VGA

    - attach to desktop

On Windows 8, in the Resolution Control Panel, set the primary display to one of the GPU, and disable the virtual monitor that is attached to the onboard VGA.